Task: Road Safety Risk Index — Karnataka

We need a per-segment road safety risk score for Karnataka's major road network, aggregated into a statewide hex grid risk surface. You will work with two layers; Karnataka.gpkg (state boundary polygon) and karnataka_major_roads.gpkg (42,475 OSM line features with fclass, maxspeed, bridge, and tunnel fields). The project, as well as both layers, must be in EPSG:32643 (WGS 84 / UTM Zone 43N).

The risk score is a weighted composite of three components computed per segment:

The first is a speed score, which uses the recorded maxspeed value where it is greater than zero (applies to 4.6% of segments), and for all other segments, impute speed from fclass using these values: motorway=100, motorway_link=80, trunk=80, trunk_link=60, primary=60, primary_link=50, secondary=50, secondary_link=40, tertiary=40, tertiary_link=30. Normalize the result by dividing by 100 to produce SPD_SCORE (0–1). Store the speed used, whether recorded or imputed, in a field called SPD_USED for traceability.

The second component is a road class score. Map each fclass to an ordinal risk weight on a 1–5 scale: motorway=5, motorway_link=4, trunk=4, trunk_link=3, primary=3, primary_link=2, secondary and secondary_link=2, tertiary and tertiary_link=1. Divide by 5 to produce CLS_SCORE (0–1).

The third component is an infrastructure hazard score (INFRA_SCORE): tunnel segments score 1.0, bridge segments score 0.6, and all other segments score 0.0. Tunnels are weighted higher than bridges due to restricted visibility and reduced escape options.

The three components must be combined into a composite RISK_SCORE, using weights of 40% for speed, 35% for road class, and 25% for infrastructure hazard. Each segment must be classified into RISK_CLS using these thresholds: Very High (≥ 0.55), High (0.35–0.549), Medium (0.20–0.349), Low (< 0.20). The scored segments, including all five computed fields, must be saved as road_risk_segments.gpkg.

To understand which parts of Karnataka are high-risk zones, a 20 km hexagonal grid is generated using create grid, with type set to Hexagon, horizontal and vertical spacing both set to 20,000 metres, and the extent set to match the Karnataka.gpkg layer in EPSG:32643 (WGS 84 / UTM Zone 43N). Since the raw grid extends slightly beyond the state boundary, it must be clipped to Karnataka.gpkg, using Clip, so boundary hexagons are trimmed to the Karnataka boundary. Spatial summary join must also be run using join by location summary, with the clipped hex grid as the input layer and road_risk_segments.gpkg as the join layer. The predicate is set to intersects so that any segment touching a cell contributes to that cell's summary. The field joined is RISK_SCORE, and the summaries requested are mean and max. This appends RISK_SCORE_mean and RISK_SCORE_max directly onto each hex cell feature.  This must be saved as risk_hex_grid.gpkg, styled with a YlOrRd graduated renderer on RISK_SCORE_mean, using RISK_SCORE_mean score thresholds: Very High (≥ 0.45), High (0.35–0.449), Medium (0.27–0.349), Low (< 0.27).

The project must deliver a final file, Road_Safety_Risk_Index_Karnataka.zip, containing the input layers (Karnataka.gpkg and karnataka_major_roads.gpkg), the output layers (road_risk_segments.gpkg and risk_hex_grid.gpkg), and a QGIS project, Road_Safety_Risk_Index_Karnataka.qgz, saved in a relative path, with all the expressions and symbology preserved, and must open without broken links.